SPOT MICRO Controller (by Mano Biletsky)

WARNING!!:

DO NOT CONNECT THE TEENSY’S USB PORT WHILE THE BOARD IS BEING POWERED BY THE “5V INPUT”

First disconnect external power, then use the USB cable for programming.

Connections:

Todo list:

√ Radio module integration in main code (remote)

√ I2C connection to Raspberry Pi (sending remote and sensor data to the Pi)

- Ultrasonic sensor integration in main code

- IMU sensor integration in main code

- Add support for servo 13,14 and 15

- Foot sensor to pin 20-23 (instead of A6-A9) (conflicting with I2C)

- Finishing Python receiver code for I2C data + integration in gait code

- Upload code to Github

Firmware:

https://github.com/adham-elarabawy/open-quadruped

Adjusted firmware with all added features from the “todo” list soon available here:

https://github.com/mano1979/spotmicro

Pin declarations:

Servo 						Teensy4.1 pin

1							2

2							3

3							4

4							5

5							6

6							7

7							8

8							9

9							10

10							11

11							12

12							13

13							24

14							25

15							28

Right Ultrasonic sensor			Teensy4.1 pin

EC 							33

TR							34

Left Ultrasonic sensor				Teensy4.1 pin

EC 							35

TR							36

Foot sensors					Teensy4.1 pin

FS1							23

FS2							22

FS3							21

FS4							20

RPI-I2C						Teensy4.1 pin

SDA							17

SCL							16

IMU-I2C						Teensy4.1 pin

SDA							18

SCL							19

INT							41

Serial (servo control)				Teensy4.1 pin

RX							0 (Serial1)

TX							1 (Serial1)

NRF24L01 radio					Teensy4.1 pin

CE							37

CSN							38

SCK							27 (SPI1)

MOSI							26 (SPI1)

MISO							39 (SPI1)

IRQ							NC

Multi purpose GPIO*				Teensy4.1 pin

MPG1						30

MPG2						31

MPG3						32

* Only available on revision 2 and newer